การสร้าง MQTT Server บน Raspberry Pi เพื่อใช้งาน Chatbot LINE ในฟาร์มอัจฉริยะ Chatbot LINE from Raspberry Pi MQTT Server for Smart Farming

ขื่อ-สกุล : B6304577 นายภานุพงศ์ แคนอินทร์

6/6 – คำถามท้ายบทเพื่อทดสอบความเข้าใจ

Quiz_101 – ทดสอบ RPi4 GPIO with Python

Python.1 - Python Switch control LED >> กดติด ปล่อยดับ

โปรแกรมที่ใช้ทดสอบ

รูป Code Capture import RPi.GPIO as GPIO # Add GPIO library to a Python sketch import time # Add time library to a Python sketch LED_pin = 35 # Ref Board SW_Pin = 37 GPIO.setmode(GPIO.BOARD) #Setup GPIO using GPIO.Pin GPIO.setup(LED_pin, GPIO.OUT) #Setup pin to output GPIO.setup(SW_Pin, GPIO.IN, pull_up_down = GPIO.PUD_UP) #Setup pin to input and Pull-Up while True: if (GPIO.input(SW_Pin)==0): # Read Botton pin GPIO.output(LED_pin,GPIO.HIGH) # Set LED pin to HIGH print("Input = 0, HIGH") else: GPIO.output(LED_pin,GPIO.LOW) # Set LED pin to LOW print("Input = 1, LOW") #time.sleep(0.5)

รูปการทดสอบ 1

รูปการทดสอบ 2

Python.2 - Python Switch control LED >> กดติด กดดับ

โปรแกรมที่ใช้ทดสอบ

รูป Code Capture import RPi.GPIO as GPIO import time LED_pin = 35 SW_Pin = 37 SW_State = 0 debounce_time = 0.1 # Adjust this value to set the debounce time (in seconds) GPIO.setmode(GPIO.BOARD) GPIO.setwarnings(False) GPIO.setup(LED_pin, GPIO.OUT) GPIO.setup(SW_Pin, GPIO.IN, pull_up_down=GPIO.PUD_UP) def toggle_led_state(): global SW_State if SW_State == 0: SW_State = 1 GPIO.output(LED_pin, GPIO.HIGH) print("State = 1, HIGH") else: SW_State = 0 GPIO.output(LED_pin, GPIO.LOW) print("State = 0, LOW") def handle_switch_press(): start_time = time.time() previous_state = GPIO.input(SW_Pin) while GPIO.input(SW_Pin) == previous_state: if time.time() - start_time >= 1.0: # Adjust the duration as needed return # Exit the function without changing the LED state time.sleep(0.1) toggle_led_state() previous_state = GPIO.HIGH current_state = GPIO.HIGH state_changed_time = time.time() while True: previous_state = current_state current_state = GPIO.input(SW_Pin) if current_state != previous_state: state_changed_time = time.time() if current_state == GPIO.LOW and time.time() - state_changed_time >= debounce_time: handle_switch_press()

รูปการทดสอบ 1

รูปการทดสอบ 2

POython.3 - Python Switch >> Switch Counter

โปรแกรมที่ใช้ทดสอบ

รูป Code Capture import RPi.GPIO as GPIO import time SW_Pin = 37 count = 0 GPIO.setmode(GPIO.BOARD) GPIO.setwarnings(False) GPIO.setup(SW_Pin, GPIO.IN, pull_up_down=GPIO.PUD_UP) def button_pressed(channel): global count count += 1 print("Count =", count) GPIO.add_event_detect(SW_Pin, GPIO.FALLING, callback=button_pressed, bouncetime=200) try: while True: time.sleep(1) except KeyboardInterrupt: GPIO.cleanup()

รูปการทดสอบ 1

รูปการทดสอบ 2

Quiz_102 – ทดสอบ RPi4 GPIO with Node-RED

Node-RED.1 – Node-RED เพื่อควบคุมสวิตซ์กดแบบ กดติด กดดับ {Switch-LED 1 คู่}

โปรแกรมที่ใช้ทดสอบ

รูป Code Capture [ { "id": "51676f5ae1675c99", "type": "tab", "label": "Q102-0101", "disabled": false, "info": "Using Node-RED (press to turn on press to turn off 1LED and 1Switch) ", "env": [] }, { "id": "eec1119ce2081360", "type": "rpi-gpio in", "z": "51676f5ae1675c99", "name": "Switch 1", "pin": "26", "intype": "up", "debounce": "25", "read": false, "bcm": true, "x": 220, "y": 300, "wires": [ [ "016c3e9be89901e3" ] ] }, { "id": "06cb6823737d2dfc", "type": "rpi-gpio out", "z": "51676f5ae1675c99", "name": "LED 1", "pin": "19", "set": "", "level": "0", "freq": "", "out": "out", "bcm": true, "x": 710, "y": 300, "wires": [] }, { "id": "016c3e9be89901e3", "type": "function", "z": "51676f5ae1675c99", "name": "State Control", "func": "context.state = context.state ¦ false;\ncontext.state = !context.state\n\nvar myContext = context.state;\nvar count = context.get(\"count\")¦¦0;\ncount += 1;\ncontext.set(\"count\",count);\nmsg.count = count;\n\nfunction isOdd(num) { \n return num % 2;\n}\n\nif(myContext === true && isOdd((count+1)/2) ===1){\n msg.payload = 1;\n return msg;\n} else if (myContext === true && isOdd((count+1)/2) ===0){\n msg.payload = 0;\n return msg;\n}", "outputs": 1, "noerr": 0, "initialize": "", "finalize": "", "libs": [], "x": 450, "y": 300, "wires": [ [ "06cb6823737d2dfc" ] ] } ]

รูปการทดสอบ 1

รูปการทดสอบ 2

Node-RED.2 - Node-RED เพื่อควบคุมสวิตซ์กดแบบ กดติด กดดับ 2 คู่

โปรแกรมที่ใช้ทดสอบ

รูป Code Capture [ { "id": "7fc9ea12076b5d25", "type": "tab", "label": "Q102-0102", "disabled": false, "info": "Using Node-RED (press to turn on press to turn off 2LED and 2Switch) ", "env": [] }, { "id": "5e0e47b70cfa15b3", "type": "rpi-gpio in", "z": "7fc9ea12076b5d25", "name": "Switch 1", "pin": "26", "intype": "up", "debounce": "25", "read": false, "bcm": true, "x": 220, "y": 300, "wires": [ [ "e9a1afc0ccd95ad2" ] ] }, { "id": "ece0b86753b4806e", "type": "rpi-gpio out", "z": "7fc9ea12076b5d25", "name": "LED 1", "pin": "19", "set": "", "level": "0", "freq": "", "out": "out", "bcm": true, "x": 690, "y": 300, "wires": [] }, { "id": "e9a1afc0ccd95ad2", "type": "function", "z": "7fc9ea12076b5d25", "name": "State Control", "func": "context.state = context.state ¦ false;\ncontext.state = !context.state\n\nvar myContext = context.state;\nvar count = context.get(\"count\")¦¦0;\ncount += 1;\ncontext.set(\"count\",count);\nmsg.count = count;\n\nfunction isOdd(num) { \n return num % 2;\n}\n\nif(myContext === true && isOdd((count+1)/2) ===1){\n msg.payload = 1;\n return msg;\n} else if (myContext === true && isOdd((count+1)/2) ===0){\n msg.payload = 0;\n return msg;\n}", "outputs": 1, "noerr": 0, "initialize": "", "finalize": "", "libs": [], "x": 450, "y": 300, "wires": [ [ "ece0b86753b4806e" ] ] }, { "id": "0c4853c44002d61b", "type": "rpi-gpio out", "z": "7fc9ea12076b5d25", "name": "LED 2", "pin": "16", "set": "", "level": "0", "freq": "", "out": "out", "bcm": true, "x": 690, "y": 400, "wires": [] }, { "id": "6efb796920ac0f04", "type": "rpi-gpio in", "z": "7fc9ea12076b5d25", "name": "Switch 2", "pin": "21", "intype": "up", "debounce": "25", "read": false, "bcm": true, "x": 220, "y": 400, "wires": [ [ "aad96eb4d550ef39" ] ] }, { "id": "aad96eb4d550ef39", "type": "function", "z": "7fc9ea12076b5d25", "name": "State Control", "func": "context.state = context.state ¦ false;\ncontext.state = !context.state\nvar myContext = context.state;\nvar count = context.get(\"count\")¦¦0;\ncount += 1;\ncontext.set(\"count\",count);\nmsg.count = count;\nfunction isOdd(num) { return num % 2;}\n\nif(myContext === true && isOdd((count+1)/2) ===1){\n msg.payload = 1;\n return msg;\n} else if (myContext === true && isOdd((count+1)/2) ===0){\n msg.payload = 0;\n return msg;\n}", "outputs": 1, "noerr": 0, "initialize": "", "finalize": "", "libs": [], "x": 450, "y": 400, "wires": [ [ "0c4853c44002d61b" ] ] } ]

รูปการทดสอบ 1

รูปการทดสอบ 2

Node-RED.3 - Node-RED เพื่ออ่าน DHT-22 Sensor

โปรแกรมที่ใช้ทดสอบ

รูป Code Capture [ { "id": "d6b35559496318d0", "type": "tab", "label": "Q102-0301", "disabled": false, "info": "", "env": [] }, { "id": "bcfc5be92bd00b6f", "type": "rpi-dht22", "z": "d6b35559496318d0", "name": "", "topic": "rpi-dht22", "dht": 22, "pintype": "0", "pin": "4", "x": 440, "y": 260, "wires": [ [ "ae17d2026071014f" ] ] }, { "id": "1ff0c6a3eade9b3c", "type": "inject", "z": "d6b35559496318d0", "name": "", "props": [ { "p": "payload" }, { "p": "topic", "vt": "str" } ], "repeat": "30", "crontab": "", "once": false, "onceDelay": 0.1, "topic": "", "payload": "", "payloadType": "date", "x": 210, "y": 260, "wires": [ [ "bcfc5be92bd00b6f" ] ] }, { "id": "ae17d2026071014f", "type": "debug", "z": "d6b35559496318d0", "name": "debug 1", "active": true, "tosidebar": true, "console": false, "tostatus": false, "complete": "true", "targetType": "full", "statusVal": "", "statusType": "auto", "x": 720, "y": 260, "wires": [] } ]

รูปการทดสอบ 1

รูปการทดสอบ 2