การสร้าง MQTT Server บน Raspberry Pi เพื่อใช้งาน Chatbot LINE ในฟาร์มอัจฉริยะ Chatbot LINE from Raspberry Pi MQTT Server for Smart Farming

ขื่อ-สกุล : B6304577 นายภานพงศ์ แคนอินทร์

6/6 - คำถามท้ายบทเพื่อทดสอบความเข้าใจ

Quiz_401 – RPi Smart Farm

- แสดงรูป โปรแกรม ของผลการทำงานตามหัวข้อ การสร้าง UI ด้วย Node-RED สำหรับฟาร์มอัจฉริยะ

Capture Node-RED Flow

Node-RED Code [ { "id": "8982e95ddaea1943", "type": "tab", "label": "Q401-0101", "disabled": false, "info": "" }, { "id": "287869c0be568bc8", "type": "function", "z": "8982e95ddaea1943", "name": "", "func": "// Tempp=38.20,Humid=73.52 \n\nvar msg1 = {};\nvar msg2 = {};\n\nvar output = msg.payload.split(\",\");\n\nvar sTempp = output[0].split(\"=\");\nmsg1.payload = sTempp[1];\nmsg1.topic = 'Temperature';\n\nvar sHumid = output[1].split(\"=\");\nmsg2.payload = sHumid[1];\nmsg2.topic = 'Humidity';\n\nreturn [msg1,msg2];", "outputs": 2, "noerr": 0, "initialize": "", "finalize": "", "libs": [], "x": 400, "y": 280, "wires": [ [ "314e6fc73de0b3b2", "ffdc2f39a719cb9f" ], [ "922989963c892994", "c87d555924e87fbc" ] ] }, { "id": "922989963c892994", "type": "ui_gauge", "z": "8982e95ddaea1943", "tab": "5ab59fbbd41d64e9", "name": "Humidity", "group": "Farm", "order": "3", "format": "{{value}}", "min": 0, "max": "100", "x": 780, "y": 300, "wires": [] }, { "id": "1ee84dc4716b94ae", "type": "ui_switch", "z": "8982e95ddaea1943", "tab": "5ab59fbbd41d64e9", "name": "LED1", "topic": "topic", "group": "Farm", "order": "0", "onvalue": "ON1", "offvalue": "OFF1", "x": 150, "y": 460, "wires": [ [ "172f630b25f47c66" ] ] }, { "id": "271f9555b85e5f30", "type": "mqtt in", "z": "8982e95ddaea1943", "name": "", "topic": "feedback/sensors", "qos": "0", "datatype": "auto-detect", "broker": "d9b6929cc83977a5", "nl": false, "rap": true, "rh": 0, "inputs": 0, "x": 190, "y": 280, "wires": [ [ "287869c0be568bc8" ] ] }, { "id": "172f630b25f47c66", "type": "mqtt out", "z": "8982e95ddaea1943", "name": "", "topic": "control/leds", "qos": "0", "retain": "false", "respTopic": "", "contentType": "", "userProps": "", "correl": "", "expiry": "", "broker": "d9b6929cc83977a5", "x": 750, "y": 460, "wires": [] }, { "id": "c87d555924e87fbc", "type": "ui_chart", "z": "8982e95ddaea1943", "tab": "5ab59fbbd41d64e9", "name": "", "group": "Farm", "order": "4", "interpolate": "linear", "nodata": "No Data", "removeOlder": 1, "removeOlderUnit": "86400", "x": 770, "y": 340, "wires": [ [], [] ] }, { "id": "314e6fc73de0b3b2", "type": "ui_chart", "z": "8982e95ddaea1943", "tab": "5ab59fbbd41d64e9", "name": "", "group": "Farm", "order": "2", "interpolate": "linear", "nodata": "No Data", "removeOlder": 1, "removeOlderUnit": "86400", "x": 770, "y": 200, "wires": [ [], [] ] }, { "id": "ffdc2f39a719cb9f", "type": "ui_gauge", "z": "8982e95ddaea1943", "tab": "5ab59fbbd41d64e9", "name": "Temperature", "group": "Farm", "order": "1", "format": "{{value}}", "min": 0, "max": "100", "x": 790, "y": 240, "wires": [] }, { "id": "5ab59fbbd41d64e9", "type": "ui_tab", "name": "Farm", "icon": "dashboard", "order": "1" }, { "id": "d9b6929cc83977a5", "type": "mqtt-broker", "name": "", "broker": "192.168.0.31", "port": "1883", "clientid": "", "autoConnect": true, "usetls": false, "protocolVersion": "4", "keepalive": "60", "cleansession": true, "birthTopic": "", "birthQos": "0", "birthRetain": "false", "birthPayload": "", "birthMsg": {}, "closeTopic": "", "closeQos": "0", "closeRetain": "false", "closePayload": "", "closeMsg": {}, "willTopic": "", "willQos": "0", "willRetain": "false", "willPayload": "", "willMsg": {}, "userProps": "", "sessionExpiry": "" } ]

Code Arduino #include <WiFi.h> #include <PubSubClient.h> #include <DHT.h> //https://www.arduinolibraries.info/libraries/dht-sensor-library #define DHT_SENSOR_PIN 4 // ESP32 pin GIOP4 connected to DHT22 sensor #define DHT_SENSOR_TYPE DHT22 #include <ArduinoJson.h> #define LED0_Pin 2 #define SW0_Pin 15 int SW0_Status = 99; unsigned long lastMsg = 0; char msg[50]; DHT dht_sensor(DHT_SENSOR_PIN, DHT_SENSOR_TYPE); const char* ssid = "KOi"; const char* password = "13062544"; const char *mqtt_broker = "192.168.0.31"; const char *mqtt_username = "mymqtt"; const char *mqtt_password = "mymqtt"; const int mqtt_port = 1883; WiFiClient espClient; PubSubClient client(espClient); void connectToWiFi() { byte mac[6]; delay(10); // We start by connecting to a WiFi network Serial.println(); Serial.print("Connecting to "); Serial.println(ssid); WiFi.mode(WIFI_STA); WiFi.begin(ssid, password); while (WiFi.status() != WL_CONNECTED) { Serial.print("."); } WiFi.macAddress(mac); Serial.println(""); Serial.println("WiFi connected"); Serial.println("IP address: "); Serial.println(WiFi.localIP()); } void connectToMQTTBroker() { byte mqttFailCount = 0; byte tooManyFailures = 10; // Loop until we're reconnected while (!client.connected()) { if (mqttFailCount <= tooManyFailures) { String client_id = "esp32-client-"; client_id += String(WiFi.macAddress()); Serial.printf("The client %s connects to the public mqtt broker\n", client_id.c_str()); if (client.connect("9z5srmKS2WAoFGYQaam1Py", mqtt_username, mqtt_password)) { Serial.println("Public Homezassistant mqtt broker connected"); // Subscribe client.subscribe("feedback/status"); client.subscribe("control/leds"); delay(100); } else { mqttFailCount ++; Serial.print("Failed. Count = "); Serial.println(mqttFailCount); Serial.println("...trying again in 5 seconds"); // Wait 5 seconds before retrying delay(5000); } } } } void MqttReceiverCallback(char* topic, byte * payload, unsigned int length) { char myPayLoad[50]; Serial.print("Message arrived ["); Serial.print(topic); Serial.print("] "); for (int i = 0; i < length; i++) { Serial.print((char)payload[i]); myPayLoad[i] = payload[i]; myPayLoad[i + 1] = '\0'; // End of String } Serial.print("\n ---> "); Serial.println(myPayLoad); myPayLoad[4] = '\0'; // String lessthan 4 Charector if ((String)myPayLoad == "ON1") digitalWrite(LED0_Pin, HIGH); if ((String)myPayLoad == "OFF1") digitalWrite(LED0_Pin, LOW); } void setup() { Serial.begin(115200); pinMode(LED0_Pin, OUTPUT); connectToWiFi(); client.setServer(mqtt_broker, mqtt_port); client.setCallback(MqttReceiverCallback); dht_sensor.begin(); // initialize the DHT sensor } void loop() { if (!client.connected()) { connectToMQTTBroker(); } client.loop(); if (digitalRead(SW0_Pin) != SW0_Status) { SW0_Status = digitalRead(SW0_Pin); Serial.println(SW0_Status == HIGH ? "Status Switch = OFF" : "Status Switch = ON"); client.publish("feedback/switch", (SW0_Status == HIGH ? "SW_OFF" : "SW_ON")); delay(100); } unsigned long now = millis(); if (now - lastMsg > 1000) { // read humidity float humi = dht_sensor.readHumidity(); // read temperature in Celsius float tempC = dht_sensor.readTemperature(); // read temperature in Fahrenheit float tempF = dht_sensor.readTemperature(true); // check whether the reading is successful or not if ( isnan(tempC) ¦¦ isnan(tempF) ¦¦ isnan(humi)) { Serial.println("Failed to read from DHT sensor!"); } else { snprintf (msg, 50, "Tempp=%0.2f,Humid=%0.2f", tempC, humi); if (client.connected()) { client.publish("feedback/sensors", msg); Serial.println(msg); Serial.println("MQTT: Send Data!!!"); Serial.println(); } } lastMsg = millis(); } }

รูปการทดสอบ 1: UI Result

รูปการทดสอบ 1

รูปการทดสอบ 2

รูปการทดสอบ 3

Quiz_402 – LINE Notify

- แสดงรูป โปรแกรม ของผลการทำงานตามหัวข้อ การส่งข้อความด้วย Node-RED สำหรับฟาร์มอัจฉริยะ

Capture Node-RED Flow mission1/4 mission 2/4 mission 3/4 mission 4/4

Node-RED Code mission1/4 [ { "id": "18068883f6581588", "type": "tab", "label": "Q402-0101", "disabled": false, "info": "" }, { "id": "a809186e12e39a1f", "type": "debug", "z": "18068883f6581588", "name": "", "active": true, "tosidebar": true, "console": false, "tostatus": false, "complete": "payload", "statusVal": "", "statusType": "auto", "x": 750, "y": 100, "wires": [] }, { "id": "209a69d8f2ab744b", "type": "function", "z": "18068883f6581588", "name": "Message", "func": "msg.token = 'FYfx1woSF3SYOZexMC7ia5jX6IaOQlO2EHL7KDeCsut';\nmsg.message= 'Hello';\nmsg.stickerPackageId = 6359;\nmsg.stickerId = 11069852;\n\nmsg.headers = {\n 'content-type':'application/x-www-form-urlencoded',\n 'Authorization':'Bearer ' + msg.token\n};\n\nmsg.payload ={\n 'message': msg.message,\n 'stickerPackageId':msg.stickerPackageId,\n'stickerId':msg.stickerId\n};\nreturn msg;\n", "outputs": 1, "noerr": 0, "initialize": "", "finalize": "", "libs": [], "x": 340, "y": 100, "wires": [ [ "d06fca3f9d1231cc" ] ] }, { "id": "6a5e1d098fbfd185", "type": "inject", "z": "18068883f6581588", "name": "Send_Sticker", "props": [ { "p": "payload" }, { "p": "topic", "vt": "str" } ], "repeat": "", "crontab": "", "once": false, "onceDelay": 0.1, "topic": "", "payload": "", "payloadType": "date", "x": 170, "y": 100, "wires": [ [ "209a69d8f2ab744b" ] ] }, { "id": "d06fca3f9d1231cc", "type": "http request", "z": "18068883f6581588", "name": "", "method": "POST", "ret": "txt", "paytoqs": "ignore", "url": "https://notify-api.line.me/api/notify", "tls": "", "persist": false, "proxy": "", "insecureHTTPParser": false, "authType": "", "senderr": false, "headers": [], "x": 530, "y": 100, "wires": [ [ "a809186e12e39a1f" ] ] } ] mission2/4 [ { "id": "bfdfb28d613836ee", "type": "tab", "label": "Q402-0201", "disabled": false, "info": "" }, { "id": "99081251955e2969", "type": "debug", "z": "bfdfb28d613836ee", "name": "", "active": true, "tosidebar": true, "console": false, "tostatus": false, "complete": "payload", "targetType": "msg", "statusVal": "", "statusType": "auto", "x": 850, "y": 80, "wires": [] }, { "id": "ca51dabcc70e764e", "type": "function", "z": "bfdfb28d613836ee", "name": "Message1", "func": "var token ='FYfx1woSF3SYOZexMC7ia5jX6IaOQlO2EHL7KDeCsut';\nvar message = 'LINE-Massage From Node-RED';\n\nmsg.headers = {\n 'content-type':'application/x-www-form-urlencoded',\n'Authorization':'Bearer ' + token\n};\n\nmsg.payload = {\n 'message': message\n};\nreturn msg;\n", "outputs": 1, "noerr": 0, "initialize": "", "finalize": "", "libs": [], "x": 370, "y": 80, "wires": [ [ "1d6974daa5523c41" ] ] }, { "id": "45c5828c24614fc5", "type": "inject", "z": "bfdfb28d613836ee", "name": "Send_Massage", "props": [ { "p": "payload" }, { "p": "topic", "vt": "str" } ], "repeat": "", "crontab": "", "once": false, "onceDelay": 0.1, "topic": "", "payload": "", "payloadType": "date", "x": 180, "y": 80, "wires": [ [ "ca51dabcc70e764e" ] ] }, { "id": "42b9851aad6c539c", "type": "inject", "z": "bfdfb28d613836ee", "name": "Send_Sticker", "props": [ { "p": "payload" }, { "p": "topic", "vt": "str" } ], "repeat": "", "crontab": "", "once": false, "onceDelay": 0.1, "topic": "", "payload": "", "payloadType": "date", "x": 170, "y": 140, "wires": [ [ "7db1785add669cde" ] ] }, { "id": "e8a63beb9ce948c9", "type": "function", "z": "bfdfb28d613836ee", "name": "Message3", "func": "\nvar url_img = 'https://bear.org/wp-content/uploads/2008/01/Griz-family-danger-ahead.jpg';\nvar token = 'FYfx1woSF3SYOZexMC7ia5jX6IaOQlO2EHL7KDeCsut';\nvar message = 'LINE-URL Image';\n\nmsg.headers = {\n 'content-type':'application/x-www-form-urlencoded',\n'Authorization':'Bearer ' + token\n};\n\nmsg.payload = {\n'message':message,\n\t'imageThumbnail':url_img,\n\t'imageFullsize':url_img\n };\n\nreturn msg;\n", "outputs": 1, "noerr": 0, "initialize": "", "finalize": "", "libs": [], "x": 370, "y": 200, "wires": [ [ "1d6974daa5523c41" ] ] }, { "id": "c6cc0446edd6c42f", "type": "inject", "z": "bfdfb28d613836ee", "name": "Send_URL-Image", "props": [ { "p": "payload" }, { "p": "topic", "vt": "str" } ], "repeat": "", "crontab": "", "once": false, "onceDelay": 0.1, "topic": "", "payload": "", "payloadType": "date", "x": 190, "y": 200, "wires": [ [ "e8a63beb9ce948c9" ] ] }, { "id": "1f1e97eda7559188", "type": "inject", "z": "bfdfb28d613836ee", "name": "Send_Image", "props": [ { "p": "payload" }, { "p": "topic", "vt": "str" } ], "repeat": "", "crontab": "", "once": false, "onceDelay": 0.1, "topic": "", "payload": "Hallo ImageTest", "payloadType": "str", "x": 170, "y": 260, "wires": [ [ "d35f0cee69c584d2" ] ] }, { "id": "7db1785add669cde", "type": "function", "z": "bfdfb28d613836ee", "name": "Message2", "func": "var token ='FYfx1woSF3SYOZexMC7ia5jX6IaOQlO2EHL7KDeCsut';\nvar message = 'LINE-Sticker';\nvar stickerPackageId =8522;\nvar stickerId = 16581266;\n\nmsg.headers = {\n 'content-type':'application/x-www-form-urlencoded',\n 'Authorization':'Bearer ' + token\n};\n\nmsg.payload = {\n 'message': message,\n'stickerPackageId':stickerPackageId,\n 'stickerId':stickerId\n};\n\nreturn msg;\n", "outputs": 1, "noerr": 0, "initialize": "", "finalize": "", "libs": [], "x": 370, "y": 140, "wires": [ [ "1d6974daa5523c41" ] ] }, { "id": "d831d3f0ce4f7c5e", "type": "NotifyImage", "z": "bfdfb28d613836ee", "name": "LINE-Image", "tmsg": "", "imgfile": "https://nypost.com/wp-content/uploads/sites/2/2021/12/cat-mad_13.jpg?quality=75&strip=all", "AccToken": "FYfx1woSF3SYOZexMC7ia5jX6IaOQlO2EHL7KDeCsut", "x": 550, "y": 260, "wires": [ [ "1d6974daa5523c41" ] ] }, { "id": "d35f0cee69c584d2", "type": "function", "z": "bfdfb28d613836ee", "name": "Message4", "func": "var message = 'Send Image Data';\nmsg.payload =message;\nreturn msg;", "outputs": 1, "noerr": 0, "initialize": "", "finalize": "", "libs": [], "x": 370, "y": 260, "wires": [ [ "d831d3f0ce4f7c5e" ] ] }, { "id": "1d6974daa5523c41", "type": "http request", "z": "bfdfb28d613836ee", "name": "", "method": "POST", "ret": "txt", "paytoqs": "ignore", "url": "https://notify-api.line.me/api/notify", "tls": "", "persist": false, "proxy": "", "insecureHTTPParser": false, "authType": "", "senderr": false, "headers": [], "x": 650, "y": 80, "wires": [ [ "99081251955e2969" ] ] } ] mission3/4 [ { "id": "6790d352.2280ac", "type": "tab", "label": "Q402-0301", "disabled": false, "info": "" }, { "id": "ca2e8cd2.5bd98", "type": "line-notify", "z": "6790d352.2280ac", "name": "", "message": "Send \"Test Massage\" from RaspberryPi", "contentType": "message", "imageThumbnail": "", "imageUrl": "", "sticker": "default", "stickerPackageId": "11", "stickerId": "1", "silent": false, "creds": "591415dad7908dc3", "x": 340, "y": 100, "wires": [ [ "aecf6b014cdb95b7" ] ] }, { "id": "b23ae221.44489", "type": "debug", "z": "6790d352.2280ac", "name": "", "active": true, "tosidebar": true, "console": false, "tostatus": false, "complete": "payload", "targetType": "msg", "statusVal": "", "statusType": "auto", "x": 730, "y": 100, "wires": [] }, { "id": "88d3a65e.a258c8", "type": "inject", "z": "6790d352.2280ac", "name": "", "props": [ { "p": "payload" }, { "p": "topic", "vt": "str" } ], "repeat": "", "crontab": "", "once": false, "onceDelay": 0.1, "topic": "", "payload": "", "payloadType": "date", "x": 140, "y": 100, "wires": [ [ "ca2e8cd2.5bd98" ] ] }, { "id": "aecf6b014cdb95b7", "type": "http request", "z": "6790d352.2280ac", "name": "", "method": "POST", "ret": "txt", "paytoqs": "ignore", "url": "https://notify-api.line.me/api/notify", "tls": "", "persist": false, "proxy": "", "insecureHTTPParser": false, "authType": "", "senderr": false, "headers": [], "x": 530, "y": 100, "wires": [ [ "b23ae221.44489" ] ] }, { "id": "591415dad7908dc3", "type": "linetoken", "name": "M4Q402" } ] mission1/4 [ { "id": "0f1e807e520403ff", "type": "tab", "label": "Q402-0401", "disabled": false, "info": "" }, { "id": "26213b1ceab8d687", "type": "line-notify", "z": "0f1e807e520403ff", "name": "", "message": "Sensor ActiveXXXXXXXXXXXXXX", "contentType": "message", "imageThumbnail": "", "imageUrl": "", "sticker": "default", "stickerPackageId": "11", "stickerId": "1", "silent": false, "creds": "591415dad7908dc3", "x": 460, "y": 240, "wires": [ [ "e759f324160a52de" ] ] }, { "id": "55c7da970b5006a2", "type": "debug", "z": "0f1e807e520403ff", "name": "", "active": true, "tosidebar": true, "console": false, "tostatus": false, "complete": "payload", "targetType": "msg", "statusVal": "", "statusType": "auto", "x": 810, "y": 280, "wires": [] }, { "id": "51ea9635d139d70e", "type": "ui_switch", "z": "0f1e807e520403ff", "tab": "4410af667a3d8692", "name": "", "topic": "topic", "group": "Q402", "order": 2, "onvalue": "ON1", "offvalue": "OFF1", "x": 90, "y": 440, "wires": [ [ "52607273423ad64e" ] ] }, { "id": "e7793cb2f893f284", "type": "function", "z": "0f1e807e520403ff", "name": "", "func": "//Tempp=38.20,Humid=73.52 \n\nvar msg1 = {};\nvar msg2 = {};\n\nvar output =msg.payload.split(\",\");\n\nvar sTempp = output[0].split(\"=\");\nmsg1.payload = sTempp[1];\nmsg1.topic= 'Temperature';\n\nvar sHumid = output[1].split(\"=\");\nmsg2.payload = sHumid[1];\nmsg2.topic ='Humidity';\n\nreturn[msg1,msg2];", "outputs": 2, "noerr": 0, "initialize": "", "finalize": "", "libs": [], "x": 320, "y": 180, "wires": [ [ "6ac6b44784427ffc", "976fac59a1d4967d", "376f66c48cfa5f5a" ], [ "107df563935a0217", "376f66c48cfa5f5a", "976fac59a1d4967d" ] ] }, { "id": "6ac6b44784427ffc", "type": "ui_gauge", "z": "0f1e807e520403ff", "tab": "4410af667a3d8692", "name": "", "group": "Q402", "order": 2, "format": "{{value}}", "min": 0, "max": "100", "x": 790, "y": 140, "wires": [] }, { "id": "976fac59a1d4967d", "type": "ui_chart", "z": "0f1e807e520403ff", "tab": "4410af667a3d8692", "name": "", "group": "Q402", "order": 1, "interpolate": "linear", "nodata": "", "removeOlder": "5", "removeOlderUnit": "60", "x": 790, "y": 80, "wires": [ [] ] }, { "id": "376f66c48cfa5f5a", "type": "ui_chart", "z": "0f1e807e520403ff", "tab": "4410af667a3d8692", "name": "", "group": "Q402", "order": 0, "interpolate": "linear", "nodata": "", "removeOlder": 1, "removeOlderUnit": "86400", "x": 790, "y": 40, "wires": [ [] ] }, { "id": "107df563935a0217", "type": "ui_gauge", "z": "0f1e807e520403ff", "tab": "4410af667a3d8692", "name": "", "group": "Q402", "order": 4, "format": "{{value}}", "min": 0, "max": "100", "x": 790, "y": 180, "wires": [] }, { "id": "e759f324160a52de", "type": "http request", "z": "0f1e807e520403ff", "name": "", "method": "POST", "ret": "txt", "paytoqs": "ignore", "url": "https://notify-api.line.me/api/notify", "tls": "", "persist": false, "proxy": "", "insecureHTTPParser": false, "authType": "", "senderr": false, "headers": [], "x": 630, "y": 280, "wires": [ [ "55c7da970b5006a2" ] ] }, { "id": "bf7aa3046b21a120", "type": "mqtt in", "z": "0f1e807e520403ff", "name": "", "topic": "feedback/sensors", "qos": "0", "datatype": "auto-detect", "broker": "d9b6929cc83977a5", "nl": false, "rap": true, "rh": 0, "inputs": 0, "x": 130, "y": 180, "wires": [ [ "e7793cb2f893f284" ] ] }, { "id": "52607273423ad64e", "type": "mqtt out", "z": "0f1e807e520403ff", "name": "", "topic": "control/leds", "qos": "0", "retain": "false", "respTopic": "", "contentType": "", "userProps": "", "correl": "", "expiry": "", "broker": "d9b6929cc83977a5", "x": 810, "y": 440, "wires": [] }, { "id": "6c5bd7c7f79e6205", "type": "mqtt in", "z": "0f1e807e520403ff", "name": "", "topic": "feedback/switch", "qos": "0", "datatype": "auto-detect", "broker": "d9b6929cc83977a5", "nl": false, "rap": true, "rh": 0, "inputs": 0, "x": 120, "y": 240, "wires": [ [ "6e39010d22ea1619" ] ] }, { "id": "b8a769625e1c9190", "type": "mqtt in", "z": "0f1e807e520403ff", "name": "", "topic": "control/leds", "qos": "0", "datatype": "auto-detect", "broker": "d9b6929cc83977a5", "nl": false, "rap": true, "rh": 0, "inputs": 0, "x": 110, "y": 320, "wires": [ [ "c5633dedfbbd146a" ] ] }, { "id": "c5633dedfbbd146a", "type": "line-notify", "z": "0f1e807e520403ff", "name": "", "message": "LED ActiveYYYYYYYYYYYYYY", "contentType": "message", "imageThumbnail": "", "imageUrl": "", "sticker": "default", "stickerPackageId": "11", "stickerId": "1", "silent": false, "creds": "591415dad7908dc3", "x": 460, "y": 320, "wires": [ [ "e759f324160a52de" ] ] }, { "id": "6e39010d22ea1619", "type": "function", "z": "0f1e807e520403ff", "name": "", "func": "\nvar stsNow =context.global.sts;\nif (msg.payload == 'SW_ON') {\n stsNow = true;\n}\nif (msg.payload == 'SW_OFF'){\n stsNow = false;\n}\ncontext.global.sts = stsNow;\nmsg.payload = stsNow;\n\nreturn msg;", "outputs": 1, "noerr": 0, "initialize": "", "finalize": "", "libs": [], "x": 320, "y": 240, "wires": [ [ "26213b1ceab8d687" ] ] }, { "id": "591415dad7908dc3", "type": "linetoken", "name": "M4Q402" }, { "id": "4410af667a3d8692", "type": "ui_tab", "name": "Q402-0401", "icon": "dashboard", "order": "4" }, { "id": "d9b6929cc83977a5", "type": "mqtt-broker", "name": "", "broker": "192.168.0.31", "port": "1883", "clientid": "", "autoConnect": true, "usetls": false, "protocolVersion": "4", "keepalive": "60", "cleansession": true, "birthTopic": "", "birthQos": "0", "birthRetain": "false", "birthPayload": "", "birthMsg": {}, "closeTopic": "", "closeQos": "0", "closeRetain": "false", "closePayload": "", "closeMsg": {}, "willTopic": "", "willQos": "0", "willRetain": "false", "willPayload": "", "willMsg": {}, "userProps": "", "sessionExpiry": "" } ]

รูปการทดสอบ 1

รูปการทดสอบ 2:

รูปการทดสอบ 3

รูปการทดสอบ 4

รูปการทดสอบ 5

Quiz_403 – LINE Chatbot

- แสดงรูป โปรแกรม ของผลการทำงานตามหัวข้อ การโต้ตอบด้วยข้อความสำหรับฟาร์มอัจฉริยะ

Capture Node-RED Flow

Node-RED Code [ { "id": "ca81a9d4127e494b", "type": "tab", "label": "Q404-0101", "disabled": false, "info": "" }, { "id": "e7622ccb4bf17cea", "type": "function", "z": "ca81a9d4127e494b", "name": "", "func": "// Tempp=38.20,Humid=73.52 \n\nvar msg1 = {};\nvar msg2 = {};\n\nvar output = msg.payload.split(\",\");\n\nvar sTempp = output[0].split(\"=\");\nmsg1.payload = sTempp[1];\nmsg1.topic = 'Temperature';\n\nvar sHumid = output[1].split(\"=\");\nmsg2.payload = sHumid[1];\nmsg2.topic = 'Humidity';\n\ncontext.global.tempp = sTempp[1];\ncontext.global.humid = sHumid[1];\n\nreturn [msg1,msg2];", "outputs": 2, "noerr": 0, "initialize": "", "finalize": "", "libs": [], "x": 300, "y": 320, "wires": [ [ "8d02bda2ce6a4e83", "056ca60eaa1e5871", "ae97a9fe27d15673" ], [ "04bf88bdb94b1023", "ae97a9fe27d15673", "056ca60eaa1e5871" ] ] }, { "id": "8d02bda2ce6a4e83", "type": "ui_gauge", "z": "ca81a9d4127e494b", "tab": "1a56f108c6e37004", "name": "", "group": "Q403-0101", "order": 2, "format": "{{value}}", "min": 0, "max": "100", "x": 650, "y": 280, "wires": [] }, { "id": "056ca60eaa1e5871", "type": "ui_chart", "z": "ca81a9d4127e494b", "tab": "1a56f108c6e37004", "name": "", "group": "Q403-0101", "order": 1, "interpolate": "linear", "nodata": "", "removeOlder": "5", "removeOlderUnit": "60", "x": 650, "y": 220, "wires": [ [] ] }, { "id": "ae97a9fe27d15673", "type": "ui_chart", "z": "ca81a9d4127e494b", "tab": "1a56f108c6e37004", "name": "", "group": "Q403-0101", "order": 0, "interpolate": "linear", "nodata": "", "removeOlder": 1, "removeOlderUnit": "86400", "x": 650, "y": 180, "wires": [ [] ] }, { "id": "04bf88bdb94b1023", "type": "ui_gauge", "z": "ca81a9d4127e494b", "tab": "1a56f108c6e37004", "name": "", "group": "Q403-0101", "order": 4, "format": "{{value}}", "min": 0, "max": "100", "x": 650, "y": 320, "wires": [] }, { "id": "bb483091a924395e", "type": "ui_switch", "z": "ca81a9d4127e494b", "tab": "1a56f108c6e37004", "name": "", "topic": "topic", "group": "Q403-0101", "order": 2, "onvalue": "ON1", "offvalue": "OFF1", "x": 290, "y": 580, "wires": [ [ "7feab23724ead6ef", "a0ce9594ad51bc5d" ] ] }, { "id": "9197b1d96b2bdcfe", "type": "function", "z": "ca81a9d4127e494b", "name": "", "func": "\nvar stsNow = context.global.sts;\nif (msg.payload == 'SW_ON') {\n stsNow = true;\n}\nif (msg.payload == 'SW_OFF') {\n stsNow = false;\n}\ncontext.global.sts = stsNow;\nmsg.payload = stsNow;\n\nreturn msg;", "outputs": 1, "noerr": 0, "initialize": "", "finalize": "", "libs": [], "x": 320, "y": 380, "wires": [ [ "8d924afac0269860" ] ] }, { "id": "8428c3554b4735a6", "type": "line-notify", "z": "ca81a9d4127e494b", "name": "", "message": "Sensor Active", "contentType": "message", "imageThumbnail": "", "imageUrl": "", "sticker": "default", "stickerPackageId": "11", "stickerId": "1", "silent": false, "creds": "591415dad7908dc3", "x": 440, "y": 480, "wires": [ [ "652ee4d0e5aac573" ] ] }, { "id": "9afd7fab7f9cb4f5", "type": "debug", "z": "ca81a9d4127e494b", "name": "", "active": true, "tosidebar": true, "console": false, "tostatus": false, "complete": "payload", "targetType": "msg", "statusVal": "", "statusType": "auto", "x": 830, "y": 480, "wires": [] }, { "id": "8d924afac0269860", "type": "switch", "z": "ca81a9d4127e494b", "name": "", "property": "payload", "propertyType": "msg", "rules": [ { "t": "eq", "v": "1", "vt": "str" } ], "checkall": "true", "repair": false, "outputs": 1, "x": 290, "y": 480, "wires": [ [ "8428c3554b4735a6" ] ] }, { "id": "a868e94c90eb9647", "type": "function", "z": "ca81a9d4127e494b", "name": "", "func": "var receive_msg = msg.payload;\nvar reply ;\n\nif(receive_msg.events[0].message.type==\"text\")\n { \n if(receive_msg.events[0].message.text==\"Hello\")\n {\n reply = {type:'text', text:\"Hello\"};\n }\n\n else if(receive_msg.events[0].message.text==\"Tempp\")\n {\n reply = {type:'text', text: context.global.tempp};\n }\n\n else if(receive_msg.events[0].message.text==\"Humid\")\n {\n reply = {type:'text', text: context.global.humid};\n }\n\n else if(receive_msg.events[0].message.text==\"Sensor\")\n { \n var xSensor = 'Sensor Off'\n if(context.global.sts){xSensor = 'Sensor On'}\n reply = {type:'text', text: xSensor};\n }\n\n else if(receive_msg.events[0].message.text==\"On\")\n {\n context.global.ctrl = 'ON1'\n reply = {type:'text', text:\"Ok turn on\"};\n }\n\n else if(receive_msg.events[0].message.text==\"Off\")\n { \n context.global.ctrl = 'OFF1'\n reply = {type:'text', text:\"Ok turn off\"};\n }\n\n else\n { \n reply = {type:'text', text:\"i don't know\"}\n }\n }\n\nvar value = [receive_msg,reply];\nmsg.payload = value;\nreturn msg;\n", "outputs": 1, "noerr": 0, "initialize": "", "finalize": "", "libs": [], "x": 360, "y": 760, "wires": [ [ "22a80689dd4116ce" ] ] }, { "id": "f6699a8a4e33ecc6", "type": "debug", "z": "ca81a9d4127e494b", "name": "", "active": true, "tosidebar": true, "console": false, "tostatus": false, "complete": "payload", "targetType": "msg", "statusVal": "", "statusType": "auto", "x": 750, "y": 760, "wires": [] }, { "id": "7feab23724ead6ef", "type": "function", "z": "ca81a9d4127e494b", "name": "", "func": "context.global.ctrl = msg.payload\nreturn msg;", "outputs": 1, "noerr": 0, "initialize": "", "finalize": "", "libs": [], "x": 600, "y": 580, "wires": [ [ "652ee4d0e5aac573" ] ] }, { "id": "326938dc1ca536d2", "type": "inject", "z": "ca81a9d4127e494b", "name": "", "props": [ { "p": "payload" }, { "p": "topic", "vt": "str" } ], "repeat": "10", "crontab": "", "once": false, "onceDelay": 0.1, "topic": "", "payload": "", "payloadType": "date", "x": 330, "y": 660, "wires": [ [ "dd26eb7aebe35092" ] ] }, { "id": "dd26eb7aebe35092", "type": "function", "z": "ca81a9d4127e494b", "name": "", "func": "msg.payload = context.global.ctrl \nreturn msg;", "outputs": 1, "noerr": 0, "initialize": "", "finalize": "", "libs": [], "x": 500, "y": 660, "wires": [ [ "a0ce9594ad51bc5d" ] ] }, { "id": "652ee4d0e5aac573", "type": "http request", "z": "ca81a9d4127e494b", "name": "", "method": "POST", "ret": "txt", "paytoqs": "ignore", "url": "https://notify-api.line.me/api/notify", "tls": "", "persist": false, "proxy": "", "insecureHTTPParser": false, "authType": "", "senderr": false, "headers": [], "x": 630, "y": 480, "wires": [ [ "9afd7fab7f9cb4f5" ] ] }, { "id": "eca20c5d08fd540d", "type": "mqtt in", "z": "ca81a9d4127e494b", "name": "", "topic": "feedback/sensors", "qos": "0", "datatype": "auto-detect", "broker": "d9b6929cc83977a5", "nl": false, "rap": true, "rh": 0, "inputs": 0, "x": 130, "y": 320, "wires": [ [ "e7622ccb4bf17cea" ] ] }, { "id": "8bcf5d4ba1886bbb", "type": "mqtt in", "z": "ca81a9d4127e494b", "name": "", "topic": "feedback/switch", "qos": "0", "datatype": "auto-detect", "broker": "d9b6929cc83977a5", "nl": false, "rap": true, "rh": 0, "inputs": 0, "x": 120, "y": 380, "wires": [ [ "9197b1d96b2bdcfe" ] ] }, { "id": "a0ce9594ad51bc5d", "type": "mqtt out", "z": "ca81a9d4127e494b", "name": "", "topic": "control/leds", "qos": "0", "retain": "false", "respTopic": "", "contentType": "", "userProps": "", "correl": "", "expiry": "", "broker": "d9b6929cc83977a5", "x": 690, "y": 660, "wires": [] }, { "id": "6cec19fea0d1b42a", "type": "http in", "z": "ca81a9d4127e494b", "name": "", "url": "/callback", "method": "post", "upload": false, "swaggerDoc": "", "x": 140, "y": 760, "wires": [ [ "a868e94c90eb9647" ] ] }, { "id": "22a80689dd4116ce", "type": "linebot-client", "z": "ca81a9d4127e494b", "name": "", "channelAccessToken": "uHe5G5d9b76TMY+4V/4ps7QcXPx+mC10MunVwNqBknT+0P9/zHCmrFFjeGfkKn+v22UUHSUudDJRcKhiBUo+fmKPx9zuDb6GmKSXw/J3jVGUsTEQEZpZVZbJJAQNHgdkTVg83niOP0RerH3Tmt7qPwdB04t89/1O/w1cDnyilFU=", "channelSecret": "bfa47238da49f53f7afc046db8263a9b", "x": 560, "y": 760, "wires": [ [ "f6699a8a4e33ecc6" ] ] }, { "id": "1a56f108c6e37004", "type": "ui_tab", "name": "Q403-0101", "icon": "dashboard", "order": "1" }, { "id": "591415dad7908dc3", "type": "linetoken", "name": "M4Q402" }, { "id": "d9b6929cc83977a5", "type": "mqtt-broker", "name": "", "broker": "192.168.0.31", "port": "1883", "clientid": "", "autoConnect": true, "usetls": false, "protocolVersion": "4", "keepalive": "60", "cleansession": true, "birthTopic": "", "birthQos": "0", "birthRetain": "false", "birthPayload": "", "birthMsg": {}, "closeTopic": "", "closeQos": "0", "closeRetain": "false", "closePayload": "", "closeMsg": {}, "willTopic": "", "willQos": "0", "willRetain": "false", "willPayload": "", "willMsg": {}, "userProps": "", "sessionExpiry": "" } ]

รูปการทดสอบ 1

รูปการทดสอบ 2:

รูปการทดสอบ 3

รูปการทดสอบ 4

รูปการทดสอบ 5

รูปการทดสอบ 6